- Deck Drainage System B-XX-XXX, Item SPV.0105.xx.

A  Description

This special provision describes furnishing, coating, and installing deck drains on railroad bridges conforming to the pertinent requirements of standard spec 514 and 612.  Included in this work are the deck drain downspouts at the substructures and all other components required for draining the bridge deck.

B  Materials

Furnish materials for the deck drains and downspouts conforming to the plans and standard spec 514.2.

Furnish deck drains that consist of perforated, half-round corrugated steel pipes and drainage pans, end closure plates, corrugated steel pipe outlet stubs, and downspouts of the sizes and gauges shown in the plans.  All components shall be galvanized and fully asphaltic coated.

Furnish downspouts and outlet stubs that consist of unperforated, corrugated steel pipes, and pipe clamps, of the sizes shown on the plans. Pipes shall be galvanized and fully asphaltic coated.

Asphaltic coating shall conform to ASHTO M190.

C  Construction

Install the deck drains conforming to the details shown on the plans and standard spec 514.3.

Set the deck drains in a thick coat of hot asphalt, mopped on the asphalt protection boards to anchor the drains in place.  Set the drains carefully to avoid gaps between sections.

Take special care not to plug the holes of the drains while coating them with asphalt. Open the holes that have become plugged, using a method that has been approved by the engineer and that will not cause damage to the coating.

Prepare shop drawings and submit them to the engineer conforming to standard spec 506.3.2.

D  Measurement

The department will measure Deck Drainage System B-XX-XXX as a single lump sum for each structure acceptably completed.

E  Payment

The department will pay for the measured quantity at the contract unit price under the following bid item:

ITEM NUMBER	DESCRIPTION	UNIT

SPV.0105.xx	Deck Drainage System B-XX-XXX	LS

Payment for Deck Drainage System B-XX-XXX is full compensation for furnishing, coating and installing deck drains, drainage pans, closure plates, outlet stubs, downspouts, pipe clamps, and all other components of the deck drainage system.

SPV.0105.xx (20260915)